Short circuit description:

Safety advice:  Kids, do not try this at home.

If you built this interface, you do it on your own risk. If you barbecue

your PC, your transceiver or anything in between including yourselves,

this is your problem! You should only built this interface if you're

sufficiently familiar with electronics. I reject any liability or

responsibility in case of damages of whatever kind - you have been warned!

I have tested the interface with a Prolific USB-to-RS-232 converter on

the PC side, and both a Yaesu VX-3 and an FT-60 transceiver, and it

works fine for me. A test with a Yaesu FT-7800 is soon to come.

The interface is fairly standard and made from parts that any decent ham

should have at home:

The supply current for the interface is drawn from the DTR line of the

RS-232 via D1. T2 together with R2 and R5 is a simple switching stage

that converts the 3,3V signal level from the transceiver to the

RxD-level. T3 with R6 acts as a pull-down switching stage for TxD.

The new thing is the decoupled, adjustable pull-up voltage that has been

realized using D2, R1, D3, P1, R3 and T1: It stabilizes the pull-up

voltage independently of the RxD and Txd-levels, different from many

other interfaces. The RxD voltage is around the voltage that DTR

delivers because it is decoupled by D2, and thus should match any serial

line. The pull-up voltage for the transceiver's two-wire interface can

be adjusted from 0-5V using P1, so that you can match almost any voltage

level. R1 has been dimensioned so that the stabilisation circuit works

with a DTR voltage greater than 7V. The voltage between Data and GND

must be adjusted to around 3,3V for my VX-3 (measure it without the

transceiver connected).

The diodes in my interface are standard 1N4001 because I had these at

hand. Any other standard diode, including 1N4148, should do the job. The

same applies to the BC-546B-transistors - they are standard small-signal

NF NPN-transistors in a TO-92 package, but I suppose any other standard

NPN type should be ok. If you intend to exchange the 5,6V Zener diode

with something else, please keep in mind that you have about 0,6 V

voltage loss in the T1-stage, and that the voltage delivered by DTR

might also be limited, especially on USB-to-serial adaptors, so I

suggest that you stick to the value of 5,6V as close as possible.

The pin numbers given are those for Sub-D-9-plug RS-232 lines. If you

have the older 25-pin plugs or anything else the pin numbers will vary!

Good luck & vy73, DL8WA